A MINI PROJECT REPORT

ON

LORA BASED MULTIHOP PREDECTIVE ENERGY DEMAND              MODELLING FOR WIRELESS COMMUNICATION NETWORKS

Submitted for partial fulfillment of the requirements for the

Award of the degree

of

BACHELOR OF TECHNOLOGY

In

ELECTRONICS AND COMMUNICATION ENGINEERING

By

Neela Varshika                23C31A04C4

Vaka RaviVardhan            23C31A04H1

Sriramula Sharavana         23C31A04G0

Sanga Ravalika                  23C31A04F4

Pulishetti Srinivas              23C31A04E3

Under the Guidance of

Mr. G. Sandeep

Assistant Professor

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

BALAJI INSTITUTE OF TECHNOLOGY AND SCIENCE

Accredited by NBA, NAAC A+ & ISO 9001:2015 Certified Institution

(Affiliated to JNTUH, Hyderabad and Approved by the AICTE, New Delhi)

NARSAMPET, WARANGAL 506 331

2025-26

BALAJI INSTITUTE OF TECHNOLOGY AND SCIENCE

(AUTONOMOUS)

Accredited by NBA, NAAC A+ & ISO 9001:2015 Certified Institution

(Affiliated to JNTUH, Hyderabad and Approved by the AICTE, New Delhi)

Laknepally, Narsampet, Warangal- 506 331

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

CERTIFICATE

This is to certify that this Project Work entitled "LORA BASED MULTIHOP PREDECTIVE ENERGY DEMAND MODELLING FOR WIRELESS COMMUNICATION NETWORKS” is a bonafide work carried out by Neela Varshika (23C3A04C4),  Vaka Ravivardhan (23C31A04H1), Sriramula Sharavana (23C31A04G0), Sanga Ravalika (23C31A04F4), Pulishetti Srinivas (23C31A04E3) in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology from JNTUH, Hyderabad during the period 2025-26 under our guidance and supervision.

Internal Guide 	                                                                                        Head of the Dept.

Mr. G. Sandeep 	 	 	 	                                                 Dr.P.Kiran Kumar Assistant Professor,                                                        	                                  Associate Professor,

Dept. of ECE                                                                                                          Dept. of ECE

External Examiner

DECLARATION

We, Neela Varshika (23C3A04C4), Vaka Ravivardhan (23C31A04H1), Sriramula Sharavana (23C31A04G0), Sanga Ravalika (23C31A04F4), Pulishetti Srinivas (23C31A04E3) hereby declare that the project entitled “LORA BASED MULTIHOP PREDECTIVE ENERGY DEMAND MODELLING FOR WIRELESS COMMUNICATION NETWORKS”, submitted in the partial fulfillment of the requirements for the award of B.Tech in Electronics and Communication Engineering to Balaji Institute of Technology & Science, Narsampet, affiliated to JNTUH, Hyderabad is a authentic work and has not been submitted to any other university or institution for award of the degree.

Neela Varshika                                 23C31A04C4

Vaka Ravi Vardhan                          23C31A04H1

Sriramula Sharavana                      23C31A04G0

Sanga Ravalika                                  23C31A04F4

Pulishetti Srinivas                            23C31A04E3

ACKNOWLEDGEMENT

It is a great pleasure to express our deepest sense of gratitude and indebtedness, to our Internal guide Mr. G. Sandeep , Assistant Professor,BITS, Narsampet, for having been a source of constant inspiration, precious guidance and generous assistance during the project work. We deem it as a privilege to have worked under his able guidance. Without his close monitoring and valuable suggestions this work wouldn’t have taken this shape. We feel that this help is un-substitutable and unforgettable.

We wish to express our sincere thanks to Dr.V.S. Hariharan, Principal, BITS, Narsampet for providing the college facilities for the completion of the project.

We are profoundly thankful to Dr. P. Kiran Kumar, Head of ECE Dept, for his cooperation and encouragement.

We are greatly thankful to our Project coordinator Dr. V.Madhu Kumar,  Assoc. Prof., Dept. of ECE, BITS, Narsampet for his support throughout our project.

Finally, we thank all the faculty members, supporting staff of ECE Dept. and friends for their kind co-operation and valuable help for completing the project.

Neela Varshika                              23C31A04C4

Vaka RaviVardhan                        23C31A04H1

Sriramula Sharavana                    23C31A04G0

Sanga Ravalika                              23C31A04F4

Pulishetti Srinivas                         23C31A04E3

ABSTRACT

A LoRa-based multi-hop wireless communication network architecture is designed to facilitate the efficient collection and transmission of IoT sensor data, combined with external environmental factors, to enable accurate predictive modeling of energy demand. The multi-hop communication paradigm overcomes the inherent range limitations of single-hop LoRa technology by allowing sensor nodes to forward data through intermediary nodes, thereby expanding network coverage and improving data reliability in geographically dispersed sensor deployments. The network architecture incorporates dynamic routing algorithms to optimize data paths and reduce packet loss, ensuring the integrity of transmitted data critical for energy demand analysis. Comprehensive exploratory data analysis (EDA) is conducted on the collected dataset comprising sensor measurements such as voltage, current, temperature, humidity, and external variables including weather conditions and temporal features. Statistical analysis and visualization techniques such as correlation heatmaps, feature distribution plots, and time series decomposition reveal underlying patterns and feature interactions relevant to energy consumption behaviors. Feature selection based on EDA outcomes enhances model efficiency and reduces computational overhead by eliminating redundant or less significant variables. For predictive classification of energy demand categories, a Support Vector Classifier (SVC) is initially employed due to its capability to handle high-dimensional feature spaces and establish clear decision boundaries. However, to address potential limitations of SVC in capturing complex non-linear relationships and to improve robustness against overfitting, a Random Forest Classifier (RFC) is implemented as the proposed model. RFC’s ensemble learning mechanism, which aggregates decisions from multiple decision trees trained on random subsets of data and features, enhances prediction stability and accuracy. Model training and validation employ stratified cross-validation techniques to ensure unbiased performance assessment. Experimental results demonstrate that RFC consistently outperforms SVC across multiple evaluation metrics including accuracy, precision, recall, and F1-score. The RFC model exhibits superior generalization on unseen data, providing more reliable predictions of energy demand fluctuations.

LIST OF FIGURES

| FIGURE NO | LIST TITLE | PAGE No |
| --- | --- | --- |
| 4.5.1 | Workflow of SVM Classifier | 16 |
| 4.5.3 | Workflow of Proposed RFC | 19 |
| 6.1 | Uploading Dataset | 29 |
| 6.2 | Count plot of energy demand categories | 29 |
| 6.3 | Correlation Heatmap | 30 |
| 6.4 | (a)Support Vector Machine Classifier (SVC) | 31 |
| 6.4 | (b) Random Forest Classifier (RFC) | 32 |
| 6.5 | Prediction on Test data using Proposed RFC | 33 |

| CONTENTS | PAGE NO |
| --- | --- |
| Declaration | I |
| Acknowledgement | ii |
| Abstract | Iii |
| List of Figures | Iv |

CHAPTER I

INTRODUCTION                                                                            	    	           1-3

1.1 Introduction

1.2 Problem Statement

1.3 Motivation

1.4 Objective

1.5 Significance

1.6 Applications

CHAPTER II

LITERATURE SURVEY							         4– 7

CHAPTER III

EXISTING SYSTEM		   	                                                                    8 – 9

3.1 Traditional System

3.2 Limitations

CHAPTER IV

PROPOSED DESIGN                                                                 		     10 – 21

4.1 Overview

4.2 Data Preprocessing

4.3 EDA

4.4 Train Test Split

4.5 Model Train and Test

4.5.1 Support Vector Machine (SVM) Classifier

4.5.2 SVM Classifier-Limitations

4.5.3 Random Forest Classifier (RFC)

4.5.4 Random Forest Classifier-Advantages

CHAPTER V

ENVIRONMENT SYSTEM                                                                           22 – 25

5.1 Software

5.2 Hardware Requirements

CHAPTER VI

results and discussions                                                                      26 – 33

6.1 Implementation description

6.2 Dataset description

6.3 Result Analysis

6.4 Comparative Analysis

CONCLUSION AND FUTURE SCOPE                                                               34

6.5 Conclusion

6.6 Future Scope

BIBLIOGRAPHY

REFERENCES                                                                                                  35– 36